| Offset | Value | Pointers/labels |
| --- | --- | --- |
| 0 | Old fp | fp |
| -4 | 6 |  |
| -8 | 7 |  |
| -12 |  | #return value |
| -16 | RA of stub |  |
| -20 |  | sp |

| Offset | Value | Pointers/labels |
| --- | --- | --- |
| 0 | Old fp |  |
| -4 | 6 |  |
| -8 | 7 |  |
| -12 |  | #RV of foo |
| -16 | RA of stub |  |
| -20 | Old fp (0) | fp |
| -24 |  | #local d |
| -28 | r0 |  |
| -32 | r1 |  |
| -36 | r2 |  |
| -40 | r3 |  |
| -44 | 6 |  |
| -48 | 7 |  |
| -52 |  | #RV of bar |
| -56 | RA of main |  |
| -60 |  | sp |

| Offset | Value | Pointers/labels |
| --- | --- | --- |
| 0 | Old fp |  |
| -4 | 6 |  |
| -8 | 7 |  |
| -12 |  | #RV of foo |
| -16 | RA of stub |  |
| -20 | Old fp (0) |  |
| -24 |  | #local d |
| -28 | r0 |  |
| -32 | r1 |  |
| -36 | r2 |  |
| -40 | r3 |  |
| -44 | 6 |  |
| -48 | 7 |  |
| -52 |  | #RV of bar |
| -56 | RA of main |  |
| -60 | Old fp (-20) | fp |
| -64 | 13 | #local e |
| -68 | r0 |  |
| -72 | r1 |  |
| -76 | r2 |  |
| -80 | r3 |  |
| -84 |  | sp |

- MV fp, sp

- JR func_main

- HALT

- func_bar:

- SW fp, 0(sp)

- MV fp, sp

- ADDI sp, sp, -4

- ADDI sp, sp, -4 # allocate space for local e

- SW r0, 0(sp)

- ADDI sp, sp, -4

- SW r1, 0(sp)

- ADDI sp, sp, -4

- SW r2, 0(sp)

- ADDI sp, sp, -4

- SW r3, 0(sp)

- ADDI sp, sp, -4

- LW r0, 16(fp)

- LW r1, 12(fp)

- ADD r2, r0, r1

- SW r2, -4(fp)

- LW r3, -4(fp)

- SW r3, 8(fp)

- J func_ret_bar

- func_ret_bar:

- ADDI sp, sp 4

- LW r3, 0(sp)

- ADDI sp, sp 4

- LW r2, 0(sp)

- ADDI sp, sp 4

- LW r1, 0(sp)

- ADDI sp, sp 4

- LW r0, 0(sp)

- MV sp, fp

- LW fp, 0(fp)

- RET

  - func_main:

  - SW fp, 0(sp)

  - MV fp, sp

  - ADDI sp, sp, -4

  - SW r0, 0(sp)

  - ADDI sp, sp, -4

  - SW r1, 0(sp)

  - ADDI sp, sp, -4

  - SW r2, 0(sp)

  - ADDI sp, sp, -4

  - SW r3, 0(sp)

  - ADDI sp, sp, -4

  - LI r0, 6

  - SW r0, 0(sp)

  - ADDI sp, sp, -4

  - LI r1, 7

  - SW r1, 0(sp)

  - ADDI sp, sp, -4

  - ADDI sp, sp, -4 # allocate space for return value

  - SW ra, 0(sp)

  - ADDI sp, sp, -4

  - JR foo

  - ADDI sp, sp, 4 # increment to stored ra of stub

  - LW ra, 0(sp)

  - ADDI sp, sp, 4 # increment to rv of foo

  - LW r2, 0(sp) # get the return value of foo into temp

  - ADDI sp, sp, 8 # increment to top parameter

  - LA r3, 0x200000000 # get address of global c

  - SW r2, 0(r3) # store return of foo in global c

  - # technically want to reload value of c into temp here (but I'm out of temps)

  - SW r2, 8(fp) # store global c as return of main

  - JR func_ret_main

  - func_ret_main:

  - ADDI sp, sp, 4

  - LW r3, 0(sp)

  - ADDI sp, sp 4

  - LW r2, 0(sp)

  - ADDI sp, sp 4

  - LW r1, 0(sp)

  - ADDI sp, sp 4

  - LW r0, 0(sp)

  - MV sp, fp

  - LW fp, 0(fp)

  - RET

  - func_foo:

  - SW fp, 0(sp)

  - MV fp, sp

  - ADDI sp, sp -4

  - ADDI sp, sp -4 # allocate space for local d

  - SW r0, 0(sp) # store temps...

  - ADDI sp, sp, -4

  - SW r1, 0(sp)

  - ADDI sp, sp, -4

  - SW r2, 0(sp)

  - ADDI sp, sp, -4

  - SW r3, 0(sp) # ...store temps

  - ADDI sp, sp, -4

  - LW r0, 16(fp) # get value of param a of foo

  - SW r0, 0(sp) # store arg a for bar

  - ADDI sp, sp -4

  - LW r1, 12(fp) # get value of param b of foo

  - SW r1, 0(sp) # store arg b for bar

  - ADDI sp, sp, -4

  - ADDI sp, sp, -4 # allocate space for rv of bar

  - SW ra, 0(sp) # store ra of previous caller

  - ADDI sp, sp, -4

  - JR func_bar

  - ADDI sp, sp 4 # increment to stored ra of previous caller

  - LW ra, 0(sp)

  - ADDI sp, sp, 4 # increment to rv of bar

  - LW r2, 0(sp) # load rv of bar into temp

  - ADDI sp, sp, 8 #increment to top arg of bar

  - # technically want to reload params a and b to pass as arguments but I'm out of registers again

  - SW r0, 0(sp) # store arg a for bar

  - ADDI sp, sp, -4

  - SW r1, 0(sp) # store arg b for bar

  - ADDI sp, sp, -4

  - ADDI sp, sp, -4 # allocate space for rv of bar

  - SW ra, 0(sp) # store ra of previous caller

  - ADDI sp, sp -4

  - JR bar

  - ADDI sp, sp, 4 # increment to ra of previous caller

  - LW ra, 0(sp)

  - ADDI sp, sp, 4 # increment to rv of bar

  - LW r3, 0(sp)

  - ADDI sp, sp, 8 # increment to top arg of bar

  - MUL r0, r0, r2 # a*(return of bar) -- out of new registers to store to

  - MUL r1, r1, r3 # b*(return of bar) -- out of new registers to store to

  - ADD r0, r0, r1

  - SW r0, -4(fp) # save result to local d

  - # technically want to reload value of d into new temp

  - SW r0, 8(fp) # store d as rv of foo

  - JR func_ret_foo

  - func_ret_foo:

  - ADDI sp, sp, 4 # increment to bottom of stored registers

  - LW r3, 0(sp) # restore registers...

  - ADDI sp, sp, 4

  - LW r2, 0(sp)

  - ADDI sp, sp, 4

  - LW r1, 0(sp)

  - ADDI sp, sp, 4

  - LW r0, 0(sp) # ...restore registers

  - MV sp, fp

  - LW fp (fp)

  - RET